# Greenhouse Gas Emissions and Climate Change

## Environmental Consequences

### Direct and Indirect Effects on Greenhouse Gas Emissions and Climate Change

#### Greenhouse Gas Emissions

Table 5.4.1-1. Carbon Dioxide Emissions and Emissions Reductions (MMTCO2) from All Passenger Cars and Light Trucks, 2027 to 2100, by Alternative a

| Alternative | Total Emissions | Emissions Reductions Compared to No-Action | Emissions Reductions Compared to Cumulative Global Emissions | Percent (%) Emissions Reductions Compared to No-Action Alternative Emissions |
| --- | --- | --- | --- | --- |
| No Action | 46,500 | - | 0.00% | 0.0% |
| PC2LT002 | 46,100 | 400 | 0.01% | 0.9% |
| PC1LT3 | 45,400 | 1,100 | 0.02% | 2.4% |
| PC2LT4 | 45,500 | 1,000 | 0.02% | 2.2% |
| PC3LT5 | 44,000 | 2,500 | 0.05% | 5.4% |
| PC6LT8 | 39,500 | 7,000 | 0.14% | 15.1% |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

MMTCO2 = million metric tons of carbon dioxide

Table 5.4.1-2. Carbon Dioxide Emissions and Emissions Reductions (MMTCO2) from All HDPUVs, 2027 to 2100, by Alternative a

| Alternative | Total Emissions | Emissions Reductions Compared to No-Action | Emissions Reductions Compared to Cumulative Global Emissions | Percent (%) Emissions Reductions Compared to No-Action Alternative Emissions |
| --- | --- | --- | --- | --- |
| No Action | 9,700 | - | 0.00% | 0.0% |
| HDPUV4 | 9,700 | - | 0.00% | 0.0% |
| HDPUV108 | 9,400 | 300 | 0.01% | 3.1% |
| HDPUV10 | 9,300 | 500 | 0.01% | 5.2% |
| HDPUV14 | 8,700 | 1,100 | 0.02% | 11.3% |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

MMTCO2 = million metric tons of carbon dioxide

Figure 5.4.1-1. Carbon Dioxide Emissions and Emissions Reductions (MMTCO2) from All Passenger Cars and Light Trucks, 2027 to 2100, by Alternative

MMTCO2 = million metric tons of carbon dioxide

Figure 5.4.1-2. Carbon Dioxide Emissions and Emissions Reductions (MMTCO2) from All HDPUVs, 2027 to 2100, by Alternative

MMTCO2 = million metric tons of carbon dioxide

Figure 5.4.1-3. Projected Annual Carbon Dioxide Emissions (MMTCO2) from CAFE Standards for All Passenger Cars and Light Trucks by Alternative

MMTCO2 = million metric tons of carbon dioxide

Figure 5.4.1-4. Projected Annual Carbon Dioxide Emissions (MMTCO2) from HDPUV Standards for All HDPUVs by Alternative

MMTCO2 = million metric tons of carbon dioxide

Table 5.4.1-3. Emissions of Greenhouse Gases (MMTCO2e per year) from All Passenger Cars and Light Trucks by Alternative a

| GHG and Year | No Action | PC2LT002 | PC1LT3 | PC2LT4 | PC3LT5 | PC6LT8 |
| --- | --- | --- | --- | --- | --- | --- |
| Carbon dioxide (CO2) |  |  |  |  |  |  |
| 2020 | 1,505 | 1,505 | 1,505 | 1,505 | 1,505 | 1,505 |
| 2040 | 833 | 822 | 803 | 797 | 770 | 694 |
| 2060 | 506 | 502 | 494 | 497 | 477 | 410 |
| 2080 | 503 | 498 | 491 | 494 | 473 | 407 |
| 2100 | 468 | 464 | 456 | 459 | 440 | 379 |
| Methane (CH4) |  |  |  |  |  |  |
| 2020 | 56 | 56 | 56 | 56 | 56 | 56 |
| 2040 | 32 | 32 | 31 | 31 | 30 | 27 |
| 2060 | 32 | 32 | 31 | 31 | 30 | 27 |
| 2080 | 22 | 22 | 21 | 21 | 21 | 18 |
| 2100 | 20 | 20 | 20 | 20 | 19 | 17 |
| Nitrous oxide (N2O) |  |  |  |  |  |  |
| 2020 | 17 | 17 | 17 | 17 | 17 | 17 |
| 2040 | 9 | 8 | 8 | 8 | 8 | 7 |
| 2060 | 5 | 5 | 5 | 5 | 4 | 4 |
| 2080 | 5 | 5 | 5 | 5 | 4 | 4 |
| 2100 | 4 | 4 | 4 | 4 | 4 | 3 |
| Total (all GHGs) |  |  |  |  |  |  |
| 2020 | 1,579 | 1,579 | 1,579 | 1,579 | 1,579 | 1,579 |
| 2040 | 873 | 862 | 842 | 836 | 808 | 728 |
| 2060 | 543 | 538 | 530 | 533 | 511 | 441 |
| 2080 | 529 | 525 | 517 | 520 | 498 | 429 |
| 2100 | 492 | 488 | 481 | 484 | 464 | 399 |

Notes:

a Emissions from 2051 to 2100 were scaled using the rate of change for the U.S. transportation fuel consumption from the SSP3-7.0 scenario. These assumptions project a slight decline over this period. Totals may not sum due to independent rounding.

MMTCO2e = million metric tons carbon dioxide equivalent

Table 5.4.1-4. Emissions of Greenhouse Gases (MMTCO2e per year) from All HDPUVs by Alternative a

| GHG and Year | No Action | HDPUV4 | HDPUV108 | HDPUV10 | HDPUV14 |
| --- | --- | --- | --- | --- | --- |
| Carbon dioxide (CO2) |  |  |  |  |  |
| 2020 | 184 | 184 | 184 | 184 | 184 |
| 2040 | 139 | 139 | 136 | 134 | 125 |
| 2060 | 126 | 126 | 121 | 118 | 109 |
| 2080 | 125 | 125 | 120 | 117 | 108 |
| 2100 | 116 | 116 | 112 | 109 | 101 |
| Methane (CH4) |  |  |  |  |  |
| 2020 | 7 | 7 | 7 | 7 | 7 |
| 2040 | 5 | 5 | 5 | 5 | 5 |
| 2060 | 5 | 5 | 5 | 5 | 5 |
| 2080 | 5 | 5 | 5 | 5 | 4 |
| 2100 | 5 | 5 | 4 | 4 | 4 |
| Nitrous oxide (N2O) |  |  |  |  |  |
| 2020 | 4 | 4 | 4 | 4 | 4 |
| 2040 | 2 | 2 | 2 | 2 | 2 |
| 2060 | 2 | 2 | 2 | 2 | 2 |
| 2080 | 2 | 2 | 2 | 2 | 2 |
| 2100 | 2 | 2 | 2 | 2 | 1 |
| Total (all GHGs) |  |  |  |  |  |
| 2020 | 194 | 194 | 194 | 194 | 194 |
| 2040 | 146 | 146 | 143 | 141 | 132 |
| 2060 | 133 | 132 | 128 | 125 | 115 |
| 2080 | 132 | 131 | 126 | 124 | 114 |
| 2100 | 122 | 122 | 118 | 115 | 106 |

Notes:

a Emissions from 2051 to 2100 were scaled using the rate of change for the U.S. transportation fuel consumption from the SSP3-7.0 scenario. These assumptions project a slight decline over this period. Totals may not sum due to independent rounding.

MMTCO2e = million metric tons carbon dioxide equivalent

##### Comparison to Annual Emissions from Passenger Cars and Light Trucks and Heavy Duty Personal Utility Vehicles

Figure 5.4.1-5. Number of Passenger Cars and Light Trucks Equivalent to Carbon Dioxide Reductions in 2035 Compared to the CAFE Standards No-Action Alternative

Figure 5.4.1-6. Number of Heavy Duty Pickup Trucks and Vans Equivalent to Carbon Dioxide Reductions in 2035 Compared to the HDPUV Standards No-Action Alternative

#### Direct and Indirect Impacts on Climate Change Indicators

Table 5.4.1-5. Comparison of MAGICC Modeling Results and Reported IPCC AR6 WG1 Results -a

| Scenario | CO2 Concentration (ppm) |  | Global Mean Increase in Surface Temperature (°C) b |  |
| --- | --- | --- | --- | --- |
|  | IPCC (2100) | MAGICC (2100) | IPCC (2081–2100) | MAGICC (2081–2100) |
| SSP1-1.9 | 337 | 337 | 1.4 | 1.3 |
| SSP1-2.6 | 446 | 446 | 1.8 | 1.8 |
| SSP2-4.5 | 603 | 603 | 2.7 | 2.8 |
| SSP3-7.0 | 867 | 867 | 3.6 | 4.0 |
| SSP5-8.5 | 1,135 | 1,135 | 4.4 | 4.9 |

Notes:

a The IPCC values represent the average of the 5 to 95 percent range of global mean surface air temperature.

Source: IPCC 2021a

ppm = parts per million; °C = degrees Celsius; MAGICC = Model for the Assessment of Greenhouse‐gas Induced Climate Change; IPCC = Intergovernmental Panel on Climate Change; SSP = Shared Socioeconomic Pathway; WGI = Working Group 1.

Table 5.4.1-6. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increase, Sea-Level Rise, and Ocean pH by CAFE Standards Alternative a

|  | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)b, c |  |  | Sea-Level Rise (cm)b, d |  |  | Ocean pHc |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 |
| Totals by Alternative |  |  |  |  |  |  |  |  |  |  |  |  |
| No Action | 490.19 | 587.76 | 838.31 | 2.0080 | 2.7883 | 4.3395 | 20.10 | 36.39 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| PC2LT002 | 490.18 | 587.74 | 838.27 | 2.0079 | 2.7882 | 4.3394 | 20.10 | 36.39 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| PC1LT3 | 490.16 | 587.70 | 838.21 | 2.0078 | 2.7880 | 4.3391 | 20.10 | 36.38 | 83.23 | 8.4014 | 8.3329 | 8.1933 |
| PC2LT4 | 490.16 | 587.70 | 838.22 | 2.0078 | 2.7880 | 4.3391 | 20.10 | 36.38 | 83.23 | 8.4014 | 8.3329 | 8.1933 |
| PC3LT5 | 490.14 | 587.64 | 838.08 | 2.0077 | 2.7878 | 4.3386 | 20.10 | 36.38 | 83.22 | 8.4014 | 8.3329 | 8.1934 |
| PC6LT8 | 490.09 | 587.46 | 837.65 | 2.0075 | 2.7870 | 4.3369 | 20.10 | 36.37 | 83.19 | 8.4014 | 8.3330 | 8.1936 |
| Reductions Under Proposed Action and Alternatives |  |  |  |  |  |  |  |  |  |  |  |  |
| PC2LT002 | 0.01 | 0.02 | 0.04 | 0.0000 | 0.0001 | 0.0001 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 |
| PC1LT3 | 0.03 | 0.06 | 0.10 | 0.0002 | 0.0003 | 0.0004 | 0.00 | 0.00 | 0.01 | 0.0000 | 0.0000 | 0.0000 |
| PC2LT4 | 0.03 | 0.06 | 0.09 | 0.0002 | 0.0003 | 0.0004 | 0.00 | 0.00 | 0.01 | 0.0000 | 0.0000 | 0.0000 |
| PC3LT5 | 0.05 | 0.12 | 0.23 | 0.0003 | 0.0006 | 0.0009 | 0.00 | 0.01 | 0.02 | 0.0000 | -0.0001 | -0.0001 |
| PC6LT8 | 0.10 | 0.30 | 0.67 | 0.0005 | 0.0013 | 0.0026 | 0.00 | 0.01 | 0.06 | -0.0001 | -0.0002 | -0.0003 |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986 to 2005.

c Values reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

d Sea-level rise changes reported as 0.00 are more than zero but less than 0.01.

CO2 = carbon dioxide; °C = degrees Celsius; ppm = parts per million; cm = centimeters

Table 5.4.1-7. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increase, Sea-Level Rise, and Ocean pH by HDPUV FE Standards Alternative a

|  | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)b, c |  |  | Sea-Level Rise (cm)b, d |  |  | Ocean pHc |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 |
| Totals by Alternative |  |  |  |  |  |  |  |  |  |  |  |  |
| No Action | 490.19 | 587.76 | 838.31 | 2.0080 | 2.7883 | 4.3395 | 20.10 | 36.39 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| HDPUV4 | 490.19 | 587.76 | 838.31 | 2.0080 | 2.7883 | 4.3395 | 20.10 | 36.39 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| HDPUV108 | 490.19 | 587.75 | 838.28 | 2.0080 | 2.7883 | 4.3394 | 20.10 | 36.39 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| HDPUV10 | 490.19 | 587.74 | 838.27 | 2.0080 | 2.7882 | 4.3394 | 20.10 | 36.39 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| HDPUV14 | 490.18 | 587.72 | 838.21 | 2.0079 | 2.7881 | 4.3391 | 20.10 | 36.38 | 83.24 | 8.4013 | 8.3328 | 8.1933 |
| Reductions Under Proposed Action and Alternatives |  |  |  |  |  |  |  |  |  |  |  |  |
| HDPUV4 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 |
| HDPUV108 | 0.00 | 0.01 | 0.03 | 0.0000 | 0.0000 | 0.0001 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 |
| HDPUV10 | 0.00 | 0.02 | 0.04 | 0.0000 | 0.0001 | 0.0002 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 |
| HDPUV14 | 0.01 | 0.04 | 0.10 | 0.0000 | 0.0002 | 0.0004 | 0.00 | 0.00 | 0.01 | 0.0000 | 0.0000 | 0.0000 |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986 to 2005.

c Values reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

d Sea-level rise changes reported as 0.00 are more than zero but less than 0.01.

CO2 = carbon dioxide; °C = degrees Celsius; ppm = parts per million; cm = centimeters

##### Atmospheric Carbon Dioxide Concentrations

Figure 5.4.1-7. CAFE Standards Reductions in Atmospheric Carbon Dioxide Concentrations Compared to the No-Action Alternative

Figure 5.4.1-8. HDPUV FE Standards Reductions in Atmospheric Carbon Dioxide Concentrations Compared to the No-Action Alternative

##### Climate Change Attributes

###### Temperature

Figure 5.4.1-9. Reductions in Global Mean Surface Temperature Compared to the CAFE Standards No-Action Alternative

Figure 5.4.1-10. HDPUV FE Standards Reductions in Global Mean Surface Temperature Compared to the HDPUV Standards No-Action Alternative

###### Sea-Level Rise

[No charts or tables]

###### Precipitation

Table 5.4.1-8. Rates of Global Mean Precipitation Increase over the 21st Century, per Shared Socioeconomic Pathways Emissions Scenario-

| Scenario | Percent per °C a,b |
| --- | --- |
| SSP8-8.5 | 1.83 |
| SSP3-7.0 | 1.71 |
| SSP2-4.5 | 2.16 |
| SSP1-2.6 | 3.05 |

Notes:

a Global percent precipitation anomalies are calculated relative to model averages over 1995–2014 for 2081–2100 from Table 4.3 in IPCC 2021.

b Percent per °C is calculated using average changes in global annual surface temperature presented in Table 5.4.2-2 scaled to the new reference time period of 1995–2014 by subtracting 0.85°C (IPCC 2021).

°C = degrees Celsius

Table 5.4.1-9. Global Mean Precipitation (Percent Increase) Using Increases in Global Mean Surface Temperature Simulated by MAGICC, by CAFE Standards Alternativea

| Scenario | 2040 | 2060 | 2100 |
| --- | --- | --- | --- |
| Global Mean Precipitation Change (scaling factor, % change in precipitation per °C change in temperature) | 1.71% |  |  |
| Global Temperature Above Average 1986–2005 Levels (°C) for the SSP3-7.0 Scenario by Alternative |  |  |  |
| No Action | 2.008 | 2.788 | 4.340 |
| PC2LT002 | 2.008 | 2.788 | 4.339 |
| PC1LT3 | 2.008 | 2.788 | 4.339 |
| PC2LT4 | 2.008 | 2.788 | 4.339 |
| PC3LT5 | 2.008 | 2.788 | 4.339 |
| PC6LT8 | 2.007 | 2.787 | 4.337 |
| Reductions in Global Temperature (°C) by Alternative (Compared to the No-Action Alternative)b |  |  |  |
| PC2LT002 | 0.000 | 0.000 | 0.000 |
| PC1LT3 | 0.000 | 0.000 | 0.000 |
| PC2LT4 | 0.000 | 0.000 | 0.000 |
| PC3LT5 | 0.000 | 0.001 | 0.001 |
| PC6LT8 | 0.001 | 0.001 | 0.003 |
| Global Mean Precipitation Increase by Alternative (%) |  |  |  |
| No Action | 3.43% | 4.77% | 7.42% |
| PC2LT002 | 3.43% | 4.77% | 7.42% |
| PC1LT3 | 3.43% | 4.77% | 7.42% |
| PC2LT4 | 3.43% | 4.77% | 7.42% |
| PC3LT5 | 3.43% | 4.77% | 7.42% |
| PC6LT8 | 3.43% | 4.77% | 7.42% |
| Reductions in Global Mean Precipitation Increase by Alternative (% Compared to the No-Action Alternative) |  |  |  |
| PC2LT002 | 0.00% | 0.00% | 0.00% |
| PC1LT3 | 0.00% | 0.00% | 0.00% |
| PC2LT4 | 0.00% | 0.00% | 0.00% |
| PC3LT5 | 0.00% | 0.00% | 0.00% |
| PC6LT8 | 0.00% | 0.00% | 0.00% |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b Precipitation changes reported as 0.000 are more than zero but less than 0.001.

c The increase in precipitation is less than 0.005%, and thus is rounded to 0.00%.

SSP = Shared Socioeconomic Pathway; MAGICC = Model for the Assessment of Greenhouse‐gas Induced Climate Change; °C = degrees Celsius

Table 5.4.1-10. Global Mean Precipitation (Percent Increase) Using Increases in Global Mean Surface Temperature Simulated by MAGICC, by HDPUV FE Standards Alternativea

| Scenario | 2040 | 2060 | 2100 |
| --- | --- | --- | --- |
| Global Mean Precipitation Change (scaling factor, % change in precipitation per °C change in temperature) | 1.71% |  |  |
| Global Temperature Above Average 1986–2005 Levels (°C) for the SSP3-7.0 Scenario by Alternative |  |  |  |
| No Action | 2.008 | 2.788 | 4.340 |
| HDPUV4 | 2.008 | 2.788 | 4.340 |
| HDPUV108 | 2.008 | 2.788 | 4.339 |
| HDPUV10 | 2.008 | 2.788 | 4.339 |
| HDPUV14 | 2.008 | 2.788 | 4.339 |
| Reductions in Global Temperature (°C) by Alternative (Compared to the No-Action Alternative)b |  |  |  |
| HDPUV4 | 0.000 | 0.000 | 0.000 |
| HDPUV108 | 0.000 | 0.000 | 0.000 |
| HDPUV10 | 0.000 | 0.000 | 0.000 |
| HDPUV14 | 0.000 | 0.000 | 0.000 |
| Global Mean Precipitation Increase by Alternative (%) |  |  |  |
| No Action | 3.43% | 4.77% | 7.42% |
| HDPUV4 | 3.43% | 4.77% | 7.42% |
| HDPUV108 | 3.43% | 4.77% | 7.42% |
| HDPUV10 | 3.43% | 4.77% | 7.42% |
| HDPUV14 | 3.43% | 4.77% | 7.42% |
| Reductions in Global Mean Precipitation Increase by Alternative (% Compared to the No-Action Alternative) |  |  |  |
| HDPUV4 | 0.00% | 0.00% | 0.00% |
| HDPUV108 | 0.00% | 0.00% | 0.00% |
| HDPUV10 | 0.00% | 0.00% | 0.00% |
| HDPUV14 | 0.00% | 0.00% | 0.00% |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values might not reflect the exact difference in all cases.

b Precipitation changes reported as 0.000 are more than zero but less than 0.001.

c The increase in precipitation is less than 0.005%, and thus is rounded to 0.00%.

SSP = Shared Socioeconomic Pathway; MAGICC = Model for the Assessment of Greenhouse‐gas Induced Climate Change; °C = degrees Celsius

###### Ocean Acidification

[No charts or tables]

##### Climate Sensitivity Variations

Table 5.4.1-11. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increases, Sea-Level Rise, and Ocean pH for Varying Climate Sensitivities for Selected CAFE Standards Alternativesa

| Alternative | Climate Sensitivity (°C for 2 × CO2) | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)b, c |  |  | Sea Level Rise (cm)d | Ocean pHc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2100 | 2100 |
| No Action | 2.4 | 450.29 | 458.64 | 422.33 | 1.4732 | 1.5505 | 1.3481 | 38.80 | 8.4332 |
|  | 3.0 | 455.64 | 469.40 | 433.77 | 1.7193 | 1.8734 | 1.6846 | 52.21 | 8.4235 |
|  | 3.9 | 461.60 | 481.57 | 454.48 | 1.9941 | 2.2416 | 2.1214 | 69.94 | 8.4065 |
| PC2LT002 | 2.4 | 450.28 | 458.63 | 422.31 | 1.4732 | 1.5504 | 1.3480 | 38.79 | 8.4332 |
|  | 3.0 | 455.63 | 469.38 | 433.74 | 1.7193 | 1.8733 | 1.6844 | 52.21 | 8.4235 |
|  | 3.9 | 461.59 | 481.55 | 454.45 | 1.9940 | 2.2415 | 2.1212 | 69.94 | 8.4066 |
| PC6LT8 | 2.4 | 450.20 | 458.38 | 421.88 | 1.4728 | 1.5493 | 1.3457 | 38.75 | 8.4336 |
|  | 3.0 | 455.54 | 469.11 | 433.25 | 1.7188 | 1.8718 | 1.6810 | 52.15 | 8.4239 |
|  | 3.9 | 461.51 | 481.29 | 453.94 | 1.9935 | 2.2398 | 2.1176 | 69.84 | 8.4070 |
| Reductions Under Alternative PC1LT3 Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC2LT002 | 2.4 | 0.01 | 0.02 | 0.02 | 0.0000 | 0.0001 | 0.0001 | 0.00 | 0.0000 |
|  | 3.0 | 0.01 | 0.02 | 0.03 | 0.0001 | 0.0001 | 0.0002 | 0.00 | 0.0000 |
|  | 3.9 | 0.01 | 0.02 | 0.03 | 0.0001 | 0.0001 | 0.0002 | 0.01 | 0.0000 |
| Reductions Under Alternative PC6LT8 Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC6LT8 | 2.4 | 0.10 | 0.26 | 0.46 | 0.0005 | 0.0012 | 0.0024 | 0.05 | -0.0004 |
|  | 3.0 | 0.10 | 0.29 | 0.52 | 0.0005 | 0.0016 | 0.0036 | 0.07 | -0.0004 |
|  | 3.9 | 0.10 | 0.28 | 0.55 | 0.0006 | 0.0018 | 0.0038 | 0.10 | -0.0004 |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986 through 2005.

cValues reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

dSea-level rise changes reported as 0.00 are more than zero but less than 0.01.

ppm = parts per million; °C = degrees Celsius; CO2 = carbon dioxide; cm = centimeters

Table 5.4.1-12. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increases, Sea-Level Rise, and Ocean pH for Varying Climate Sensitivities for Selected HDPUV Standards Alternativesa

| Alternative | Climate Sensitivity (°C for 2 × CO2) | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)b, c |  |  | Sea Level Rise (cm)d | Ocean pHc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2100 | 2100 |
| No Action | 2.4 | 450.29 | 458.64 | 422.33 | 1.4732 | 1.5505 | 1.3481 | 38.80 | 8.4332 |
|  | 3.0 | 455.64 | 469.40 | 433.77 | 1.7193 | 1.8734 | 1.6846 | 52.21 | 8.4235 |
|  | 3.9 | 461.60 | 481.57 | 454.48 | 1.9941 | 2.2416 | 2.1214 | 69.94 | 8.4065 |
| HDPUV4 | 2.4 | 450.29 | 458.64 | 422.33 | 1.4732 | 1.5505 | 1.3481 | 38.80 | 8.4332 |
|  | 3.0 | 455.64 | 469.40 | 433.77 | 1.7193 | 1.8734 | 1.6846 | 52.21 | 8.4235 |
|  | 3.9 | 461.60 | 481.57 | 454.48 | 1.9941 | 2.2416 | 2.1214 | 69.94 | 8.4065 |
| HDPUV14 | 2.4 | 450.29 | 458.61 | 422.26 | 1.4732 | 1.5504 | 1.3477 | 38.79 | 8.4332 |
|  | 3.0 | 455.63 | 469.36 | 433.69 | 1.7193 | 1.8732 | 1.6840 | 52.21 | 8.4236 |
|  | 3.9 | 461.60 | 481.54 | 454.40 | 1.9941 | 2.2414 | 2.1208 | 69.93 | 8.4066 |
| Reductions Under Alternative HDPUV4 Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| HDPUV4 | 2.4 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 | 0.00 | 0.0000 |
|  | 3.0 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 | 0.00 | 0.0000 |
|  | 3.9 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 | 0.00 | 0.0000 |
| Reductions Under Alternative HDPUV14 Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| HDPUV14 | 2.4 | 0.01 | 0.03 | 0.07 | 0.0000 | 0.0001 | 0.0004 | 0.01 | -0.0001 |
|  | 3.0 | 0.01 | 0.04 | 0.08 | 0.0000 | 0.0002 | 0.0005 | 0.01 | -0.0001 |
|  | 3.9 | 0.01 | 0.04 | 0.08 | 0.0000 | 0.0002 | 0.0006 | 0.01 | -0.0001 |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986 through 2005.

cValues reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

dSea-level rise changes reported as 0.00 are more than zero but less than 0.01.

ppm = parts per million; °C = degrees Celsius; CO2 = carbon dioxide; cm = centimeters

### Cumulative Impacts on Greenhouse Gas Emissions and Climate Change

#### Greenhouse Gas Emissions

Table 5.4.2-1. Carbon Dioxide Emissions and Emissions Reductions (MMTCO2) from All LD Vehicles and HDPUVs, 2027 to 2100, by Alternative a

| Alternative | Total Emissions | Emissions Reductions Compared to No Action | Emissions Reductions Compared to Cumulative Global Emissions | Percent (%) Emissions Reductions Compared to No-Action Alternative Emissions |
| --- | --- | --- | --- | --- |
| No Action | 56,200 | - | 0.00% | -- |
| PC2LT002 + HDPUV4 | 55,700 | 500 | 0.01% | 0.9% |
| PC2LT002 + HDPUV108 | 55,400 | 800 | 0.02% | 1.4% |
| PC6LT8 + HDPUV14 | 45,700 | 10,500 | 0.21% | 18.7% |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

MMTCO2 = million metric tons of carbon dioxide

Figure 5.4.2-1. Carbon Dioxide Emissions and Emissions Reductions (MMTCO2) from All LD Vehicles and HDPUVs, 2027 to 2100, by Alternative

MMTCO2 = million metric tons of carbon dioxide

Figure 5.4.2-2. Projected Annual Carbon Dioxide Emissions (MMTCO2) from All LD Vehicles and HDPUVs by Alternative

MMTCO2 = million metric tons of carbon dioxide

Table 5.4.2-2. Emissions of Greenhouse Gases (MMTCO2e per year) from All LD Vehicles and HDPUVs by Alternative a

| GHG and Year | No Action | PC2LT002 + HDPUV4 | PC2LT002 + HDPUV108 | PC6LT8 + HDPUV14 |
| --- | --- | --- | --- | --- |
| Carbon dioxide (CO2) |  |  |  |  |
| 2020 | 1,689 | 1,689 | 1,689 | 1,689 |
| 2040 | 972 | 960 | 958 | 786 |
| 2060 | 632 | 626 | 621 | 481 |
| 2080 | 628 | 622 | 617 | 478 |
| 2100 | 584 | 578 | 574 | 444 |
| Methane (CH4) |  |  |  |  |
| 2020 | 63 | 63 | 63 | 63 |
| 2040 | 37 | 37 | 37 | 31 |
| 2060 | 37 | 37 | 37 | 31 |
| 2080 | 27 | 26 | 26 | 22 |
| 2100 | 25 | 25 | 24 | 20 |
| Nitrous oxide (N2O) |  |  |  |  |
| 2020 | 21 | 21 | 21 | 21 |
| 2040 | 11 | 11 | 11 | 9 |
| 2060 | 7 | 7 | 6 | 5 |
| 2080 | 7 | 6 | 6 | 5 |
| 2100 | 6 | 6 | 6 | 4 |
| Total (all GHGs) |  |  |  |  |
| 2020 | 1,773 | 1,773 | 1,773 | 1,773 |
| 2040 | 1,020 | 1,008 | 1,005 | 826 |
| 2060 | 676 | 669 | 664 | 517 |
| 2080 | 661 | 654 | 649 | 504 |
| 2100 | 615 | 609 | 604 | 469 |

Notes:

a Emissions from 2051 to 2100 were scaled using the rate of change for the U.S. transportation fuel consumption from the SSP3-7.0 scenario. These assumptions project a slight decline over this period. Totals may not sum due to independent rounding.

MMTCO2e = million metric tons carbon dioxide equivalent

#### Cumulative Impacts on Climate Change Indicators

Table 5.4.2-3. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increase, and Sea-Level Rise, and Ocean pH for Selected Combined Impacts a

| Combined Impacts | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)b,c |  |  | Sea-Level Rise (cm)b |  |  | Ocean pHc |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 |
| No Action | 472.65 | 532.40 | 587.78 | 1.8517 | 2.3699 | 2.8264 | 19.17 | 33.85 | 67.12 | 8.4149 | 8.3704 | 8.3328 |
| PC2LT002 + HDPUV4 | 472.64 | 532.38 | 587.74 | 1.8517 | 2.3698 | 2.8262 | 19.17 | 33.85 | 67.11 | 8.4149 | 8.3704 | 8.3328 |
| PC2LT002 + HDPUV108 | 472.64 | 532.37 | 587.71 | 1.8517 | 2.3698 | 2.8261 | 19.17 | 33.85 | 67.11 | 8.4149 | 8.3704 | 8.3329 |
| PC6LT8 + HDPUV14 | 472.53 | 531.99 | 586.89 | 1.8512 | 2.3682 | 2.8222 | 19.17 | 33.84 | 67.03 | 8.4150 | 8.3707 | 8.3334 |
| Reductions Under Alternatives |  |  |  |  |  |  |  |  |  |  |  |  |
| PC2LT002 + HDPUV4 | 0.01 | 0.02 | 0.04 | 0.0001 | 0.0001 | 0.0002 | 0.00 | 0.00 | 0.00 | 0.0000 | 0.0000 | 0.0000 |
| PC2LT002 + HDPUV108 | 0.01 | 0.03 | 0.07 | 0.0000 | 0.0001 | 0.0003 | 0.00 | 0.00 | 0.01 | 0.0000 | 0.0000 | 0.0000 |
| PC6LT8 + HDPUV14 | 0.12 | 0.41 | 0.89 | 0.0005 | 0.0018 | 0.0042 | 0.00 | 0.02 | 0.08 | -0.0001 | -0.0003 | -0.0006 |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986–2005.

c Values reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

CO2 = carbon dioxide; ppm = parts per million; °C = degrees Celsius; cm = centimeters

##### Atmospheric Carbon Dioxide Concentrations

Figure 5.4.2-3. Combined Impacts Reductions in Atmospheric Carbon Dioxide Concentrations Compared to the No-Action Alternatives

##### Temperature

Figure 5.4.2-4. Combined Impacts Reductions in Global Mean Surface Temperature Compared to the No-Action Alternatives

##### Precipitation

Table 5.4.2-4. Global Mean Precipitation (Percent Increase) Using Increases in Global Mean Surface Temperature Simulated by MAGICC, by Combined Impactsa

| Scenario | 2040 | 2060 | 2100 |
| --- | --- | --- | --- |
| Global Mean Precipitation Change (scaling factor, % change in precipitation per °C change in temperature) | 2.16% |  |  |
| Global Temperature Above Average 1986–2005 Levels (°C) for the SSP2-4.5 Scenario |  |  |  |
| No Action | 1.852 | 2.370 | 2.826 |
| PC2LT002 + HDPUV4 | 1.852 | 2.370 | 2.826 |
| PC2LT002 + HDPUV108 | 1.852 | 2.370 | 2.826 |
| PC6LT8 + HDPUV14 | 1.851 | 2.368 | 2.822 |
| Reductions in Global Temperature (°C) Compared to the No-Action Alternativeb |  |  |  |
| PC2LT002 + HDPUV4 | 0.000 | 0.000 | 0.000 |
| PC2LT002 + HDPUV108 | 0.000 | 0.000 | 0.000 |
| PC6LT8 + HDPUV14 | 0.001 | 0.002 | 0.004 |
| Global Mean Precipitation Increase (%) |  |  |  |
| No Action | 4.00% | 5.12% | 6.11% |
| PC2LT002 + HDPUV4 | 4.00% | 5.12% | 6.10% |
| PC2LT002 + HDPUV108 | 4.00% | 5.12% | 6.10% |
| PC6LT8 + HDPUV14 | 4.00% | 5.12% | 6.10% |
| Reductions in Global Mean Precipitation Increase Compared to the No-Action Alternativec |  |  |  |
| PC2LT002 + HDPUV4 | 0.00% | 0.00% | 0.00% |
| PC2LT002 + HDPUV108 | 0.00% | 0.00% | 0.00% |
| PC6LT8 + HDPUV14 | 0.00% | 0.00% | 0.01% |

Notes:

a The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

b Precipitation changes reported as 0.000 are more than zero but less than 0.001.

c The increase in precipitation is less than 0.005% and thus is rounded to 0.00%.

SSP = Shared Socioeconomic Pathway; MAGICC = Model for the Assessment of Greenhouse‐gas Induced Climate Change;

°C = degrees Celsius

##### Sea-Level Rise

[No charts or tables]

##### Ocean Acidification

[No charts or tables]

##### Climate Sensitivity Variations

Table 5.4.2-5. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increases, Sea-Level Rise,a and Ocean pH for SSP1-2.6 for Selected Combined Impactsb

| Combined Impacts | Climate Sensitivity (°C for 2 × CO2) | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)c, d |  |  | Sea-Level Rise (cm)c, e | Ocean pHd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2100 | 2100 |
| No Action | 2.4 | 450.29 | 458.64 | 422.33 | 1.4732 | 1.5505 | 1.3481 | 38.80 | 8.4563 |
|  | 3.0 | 455.64 | 469.40 | 433.77 | 1.7193 | 1.8734 | 1.6846 | 52.21 | 8.4465 |
|  | 3.9 | 461.60 | 481.57 | 454.48 | 1.9941 | 2.2416 | 2.1214 | 69.94 | 8.4294 |
| PC2LT002 + HDPUV4 | 2.4 | 450.28 | 458.62 | 422.30 | 1.4732 | 1.5504 | 1.3479 | 38.79 | 8.4563 |
|  | 3.0 | 455.63 | 469.37 | 433.74 | 1.7193 | 1.8732 | 1.6843 | 52.21 | 8.4465 |
|  | 3.9 | 461.59 | 481.55 | 454.45 | 1.9940 | 2.2414 | 2.1211 | 69.94 | 8.4294 |
| PC6LT8 + HDPUV14 | 2.4 | 450.17 | 458.27 | 421.65 | 1.4727 | 1.5488 | 1.3445 | 38.73 | 8.4569 |
|  | 3.0 | 455.52 | 468.99 | 433.00 | 1.7187 | 1.8711 | 1.6792 | 52.12 | 8.4472 |
|  | 3.9 | 461.49 | 481.17 | 453.66 | 1.9935 | 2.2391 | 2.1156 | 69.80 | 8.4300 |
| Reductions Under Low CAFE + Low HDPUV Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC2LT002 + HDPUV4 | 2.4 | 0.01 | 0.02 | 0.03 | 0.0000 | 0.0001 | 0.0002 | 0.00 | 0.0000 |
|  | 3.0 | 0.01 | 0.02 | 0.04 | 0.0001 | 0.0001 | 0.0003 | 0.01 | 0.0000 |
|  | 3.9 | 0.01 | 0.02 | 0.04 | 0.0001 | 0.0001 | 0.0003 | 0.01 | 0.0000 |
| Reductions Under High CAFE + High HDPUV Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC6LT8 + HDPUV14 | 2.4 | 0.12 | 0.38 | 0.69 | 0.0005 | 0.0017 | 0.0035 | 0.07 | -0.0006 |
|  | 3.0 | 0.12 | 0.41 | 0.78 | 0.0001 | 0.0001 | 0.0003 | 0.10 | -0.0006 |
|  | 3.9 | 0.12 | 0.40 | 0.82 | 0.0006 | 0.0025 | 0.0058 | 0.14 | -0.0007 |

Notes:

a Sea-level rise results are based on the regression analysis described in Section 5.3.3, Methods for Estimating Climate Effects.

b The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

c The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986–2005.

d Values reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

eSea-level rise changes reported as 0.00 are more than zero but less than 0.01.

ppm = parts per million; °C = degrees Celsius; CO2 = carbon dioxide; cm = centimeters; SSP = Shared Socioeconomic Pathway

Table 5.4.2-6. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increases, Sea-Level Rise,a and Ocean pH for SSP2-4.5 for Selected Combined Impactsb

| Combined Impacts | Climate Sensitivity (°C for 2 × CO2) | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)c, d |  |  | Sea-Level Rise (cm)c, e | Ocean pHd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2100 | 2100 |
| No Action | 2.4 | 467.16 | 518.61 | 558.07 | 1.5970 | 2.0146 | 2.2976 | 51.76 | 8.3526 |
|  | 3.0 | 472.65 | 532.40 | 587.78 | 1.8517 | 2.3699 | 2.8264 | 67.12 | 8.3328 |
|  | 3.9 | 478.62 | 546.17 | 617.33 | 2.1320 | 2.8074 | 3.4269 | 86.81 | 8.3140 |
| PC2LT002 + HDPUV4 | 2.4 | 467.15 | 518.59 | 558.03 | 1.5969 | 2.0145 | 2.2974 | 51.76 | 8.3526 |
|  | 3.0 | 472.64 | 532.38 | 587.74 | 1.8517 | 2.3698 | 2.8262 | 67.11 | 8.3328 |
|  | 3.9 | 478.61 | 546.14 | 617.28 | 2.1320 | 2.8073 | 3.4266 | 86.80 | 8.3140 |
| PC6LT8 + HDPUV14 | 2.4 | 467.04 | 518.22 | 557.27 | 1.5965 | 2.0129 | 2.2943 | 51.70 | 8.3531 |
|  | 3.0 | 472.53 | 531.99 | 586.89 | 1.8512 | 2.3682 | 2.8222 | 67.03 | 8.3334 |
|  | 3.9 | 478.50 | 545.74 | 616.41 | 2.1315 | 2.8052 | 3.4214 | 86.69 | 8.3146 |
| Reductions Under Low CAFE + Low HDPUV Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC2LT002 + HDPUV4 | 2.4 | 0.01 | 0.02 | 0.04 | 0.0000 | 0.0001 | 0.0002 | 0.00 | 0.0000 |
|  | 3.0 | 0.01 | 0.02 | 0.04 | 0.0001 | 0.0001 | 0.0002 | 0.00 | 0.0000 |
|  | 3.9 | 0.01 | 0.02 | 0.04 | 0.0001 | 0.0001 | 0.0003 | 0.01 | 0.0000 |
| Reductions Under High CAFE + High HDPUV Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC6LT8 + HDPUV14 | 2.4 | 0.12 | 0.39 | 0.80 | 0.0005 | 0.0017 | 0.0033 | 0.06 | -0.0005 |
|  | 3.0 | 0.12 | 0.41 | 0.89 | 0.0005 | 0.0018 | 0.0042 | 0.08 | -0.0006 |
|  | 3.9 | 0.12 | 0.42 | 0.92 | 0.0006 | 0.0022 | 0.0055 | 0.12 | -0.0006 |

Notes:

a Sea-level rise results are based on the regression analysis described in Section 5.3.3, Methods for Estimating Climate Effects.

b The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

c The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986–2005.

d Values reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

eSea-level rise changes reported as 0.00 are more than zero but less than 0.01.

ppm = parts per million; °C = degrees Celsius; CO2 = carbon dioxide; cm = centimeters; SSP = Shared Socioeconomic Pathway

Table 5.4.2-7. Carbon Dioxide Concentrations, Global Mean Surface Temperature Increases, Sea-Level Rise,a and Ocean pH for SSP3-7.0 for Selected Combined Impactsb

| Combined Impacts | Climate Sensitivity (°C for 2 × CO2) | CO2 Concentration (ppm) |  |  | Global Mean Surface Temperature Increase (°C)c, d |  |  | Sea-Level Rise (cm)c, e | Ocean pHd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2040 | 2060 | 2100 | 2040 | 2060 | 2100 | 2100 | 2100 |
| No Action | 2.4 | 484.64 | 572.03 | 791.90 | 1.7365 | 2.3897 | 3.6101 | 65.72 | 8.2162 |
|  | 3.0 | 490.19 | 587.76 | 838.31 | 2.0080 | 2.7883 | 4.3395 | 83.24 | 8.1933 |
|  | 3.9 | 496.65 | 604.77 | 886.21 | 2.3083 | 3.2924 | 5.2022 | 106.54 | 8.1707 |
| PC2LT002 + HDPUV4 | 2.4 | 484.63 | 572.00 | 791.85 | 1.7365 | 2.3896 | 3.6100 | 65.72 | 8.2162 |
|  | 3.0 | 490.18 | 587.73 | 838.26 | 2.0079 | 2.7882 | 4.3394 | 83.24 | 8.1933 |
|  | 3.9 | 496.64 | 604.74 | 886.16 | 2.3082 | 3.2923 | 5.2020 | 106.54 | 8.1707 |
| PC6LT8 + HDPUV14 | 2.4 | 484.52 | 571.62 | 790.98 | 1.7361 | 2.3880 | 3.6072 | 65.67 | 8.2166 |
|  | 3.0 | 490.07 | 587.34 | 837.31 | 2.0074 | 2.7864 | 4.3357 | 83.17 | 8.1937 |
|  | 3.9 | 496.52 | 604.34 | 885.13 | 2.3077 | 3.2902 | 5.1979 | 106.44 | 8.1712 |
| Reductions Under Low CAFE + Low HDPUV Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC2LT002 + HDPUV4 | 2.4 | 0.01 | 0.02 | 0.04 | 0.0000 | 0.0001 | 0.0001 | 0.00 | 0.0000 |
|  | 3.0 | 0.01 | 0.02 | 0.05 | 0.0000 | 0.0001 | 0.0002 | 0.00 | 0.0000 |
|  | 3.9 | 0.01 | 0.02 | 0.05 | 0.0001 | 0.0001 | 0.0002 | 0.01 | 0.0000 |
| Reductions Under High CAFE + High HDPUV Compared to the No-Action Alternative |  |  |  |  |  |  |  |  |  |
| PC6LT8 + HDPUV14 | 2.4 | 0.12 | 0.41 | 0.92 | 0.0005 | 0.0017 | 0.0029 | 0.06 | -0.0005 |
|  | 3.0 | 0.12 | 0.42 | 1.00 | 0.0006 | 0.0019 | 0.0039 | 0.08 | -0.0005 |
|  | 3.9 | 0.13 | 0.43 | 1.08 | 0.0006 | 0.0022 | 0.0042 | 0.10 | -0.0005 |

Notes:

a Sea-level rise results are based on the regression analysis described in Section 5.3.3, Methods for Estimating Climate Effects.

b The numbers in this table have been rounded for presentation purposes. As a result, the values do not reflect the exact differences between the values.

c The values for global mean surface temperature and sea-level rise are relative to the average of the years 1986–2005.

d Values reported as 0.0000 are more than zero but less than 0.0001. Values reported as -0.0000 are less than zero but more than -0.0001.

eSea-level rise changes reported as 0.00 are more than zero but less than 0.01.

ppm = parts per million; °C = degrees Celsius; CO2 = carbon dioxide; cm = centimeters; SSP = Shared Socioeconomic Pathway