Occupant Model Checklist

Toyota Venza	June 2014

Checklist for Running the Toyota Venza Occupant Model

- Determine the firing time using the “5-30” rule.

  - Double integrate the crash pulse to get the displacement time history.

  - Identify the time corresponding to 5 inches of displacement.

  - Subtract 30 ms from that time to get the firing time.

- Change the value of the following variables under CONTROL_ANALYSIS.TIME to the calculated firing time.

  - DAB_TTF

  - Anchoragepretensioner_TTF

  - Anchoragebuckleframe_TTF

  - Ret_stroke_ttf

- Change the INITIAL.JOINT_VEL for Motion_jnt (under the DS3_Interiors system) to correspond to the desired initial velocity.

- Change the crash pulse using the following functions, which are called in MOTION.JOINT_ACC for Motion_jnt (under the DS3_Interiors system).

  - For full frontal

    - /DS3_Interiors/vehicle_acc

  - For offset or centerline pole

    - /DS3_Interiors/vehicle_acc

    - /DS3_Interiors/vehicle_accY

    - /DS3_Interiors/vehicle_accZ

- If PSM is enabled in the model, update the PSM file name under /DS3_Interiors/Venza_Interior/MOTION.STRUCT_DISP